LAB GUIDE

# Capturing IEEE 802.15.4 Packets
with an nRF52840 Dongle

macOS • Nordic serial sniffer • Python packet logging

| Target hardware | nRF52840 USB Dongle (PCA10059) |
| --- | --- |
| Host system | macOS |
| Serial port | /dev/cu.usbmodem11301 |
| Radio channel | IEEE 802.15.4 channel 15 |

Lab outcome: Configure the dongle, verify the USB serial interface, capture 802.15.4 frames, and preserve timestamped packet data in a text file.

# 1. Learning Objectives

Verify that macOS recognizes the nRF52840 dongle and identifies its serial device.

Configure the sniffer for a selected IEEE 802.15.4 channel.

Capture and observe frame bytes, received signal strength (RSSI), and link quality (LQI).

Record captured packets in a text file for later analysis.

Diagnose common port, Python, firmware, and channel-selection problems.

# 2. Required Equipment and Software

| Item | Requirement | Purpose |
| --- | --- | --- |
| Radio hardware | nRF52840 Dongle (PCA10059) | Receives over-the-air 802.15.4 frames |
| Firmware | Nordic IEEE 802.15.4 serial sniffer firmware | Provides channel, receive, and sleep commands |
| Host | Mac with an available USB port | Runs the Python capture program |
| Python | Python 3 with pyserial | Controls and reads the serial port |
| Script | nrf802154_serial_sniffer.py | Parses and displays received frames |
| Optional | Wireshark 4.0 or later | Detailed protocol analysis |

# 3. Important Firmware Compatibility Note

Required interface: This lab's Python program expects firmware that accepts the text commands “channel <11–26>”, “receive”, and “sleep”, and reports packets as “received: <hex> power: <dBm> lqi: <value> time: <timestamp>”.

If the dongle emits binary data or does not respond to these commands, it may contain Nordic's binary Extcap firmware. That firmware should be used through the official nRF Sniffer Extcap utility, or replaced with the command-line sniffer sample before performing this lab.

# 4. Procedure

## Part A — Verify USB Detection

Connect the dongle.

Insert the nRF52840 dongle directly into the Mac or into a reliable data-capable USB adapter.

List Nordic serial devices.

Open Terminal and execute:

ls -l /dev/cu.usb* /dev/tty.usb* 2>/dev/null

Expected result:

/dev/cu.usbmodem11301
/dev/tty.usbmodem11301

Use this device: /dev/cu.usbmodem11301. On macOS, the /dev/cu.* device is normally preferred for an outbound serial connection.

Check USB enumeration if necessary.

If no serial device appears, inspect the USB device list:

system_profiler SPUSBDataType | grep -A 12 -i -E "Nordic|nRF|Open DFU"

## Part B — Prepare Python

Confirm Python 3.

Run the following command. Python 3.10 or later is recommended for Nordic's current tooling.

python3 --version

Install pyserial.

Install the library used to communicate with the USB serial port:

python3 -m pip install --user pyserial

Verify the installation.

Confirm that the same Python interpreter can import the serial module:

python3 -c "import serial; print(serial.__version__)"

## Part C — Place and Inspect the Script

Copy the program to a working folder.

Place nrf802154_serial_sniffer.py in a folder where you want the packet log to be created, then change to that folder in Terminal.

cd /path/to/your/lab/folder

Review command options.

Display the built-in help message:

python3 nrf802154_serial_sniffer.py --help

## Part D — Capture Packets

Start the transmitter and receiver.

Configure both test nodes for IEEE 802.15.4 channel 15. For the current OzturkLab test network, the PAN ID is 0x1234.

Run the sniffer and save the console output.

The tee command displays packets in Terminal while simultaneously writing them to captured_packets.txt:

python3 nrf802154_serial_sniffer.py \
  --port /dev/cu.usbmodem11301 \
  --channel 15 | tee captured_packets.txt

Observe packet reception.

A successfully parsed frame is displayed with the host UTC timestamp, byte length, RSSI, LQI, and hexadecimal frame data.

2026-09-05T05:12:03+00:00  len=25  RSSI=-48 dBm  LQI=220  6188013412030002...

Stop the capture.

Press Control-C. The program sends the sleep command before closing the serial connection.

## Part E — Preserve Additional Capture Sessions

The standard tee command replaces an existing file. To append a later session to the same file, add the -a option:

python3 nrf802154_serial_sniffer.py \
  --port /dev/cu.usbmodem11301 \
  --channel 15 | tee -a captured_packets.txt

For machine-readable output, use the program's JSON Lines option. Each packet becomes one JSON object with separate fields:

python3 nrf802154_serial_sniffer.py \
  --port /dev/cu.usbmodem11301 \
  --channel 15 \
  --jsonl captured_packets.jsonl

# 5. Understanding the Output

| Field | Meaning | Interpretation |
| --- | --- | --- |
| captured_at | UTC time recorded by the Mac | Useful for correlating events across logs |
| length | Number of captured frame bytes | Check against the application's expected frame size |
| RSSI | Received signal strength in dBm | Values closer to 0 indicate stronger received power |
| LQI | Link Quality Indicator | Higher values generally indicate a cleaner received frame |
| device_time | Timestamp maintained by the dongle | Useful for relative packet timing |
| frame_hex | Raw 802.15.4 frame bytes | Contains MAC header, addresses, payload, and possibly FCS |

# 6. Verification Checklist

☐ The dongle appears as /dev/cu.usbmodem11301 or another /dev/cu.usbmodem* device.

☐ The serial port opens without a permission or 'device busy' error.

☐ The script reports that it is listening on channel 15.

☐ At least one received frame is displayed with RSSI and LQI.

☐ captured_packets.txt exists and contains the displayed packet lines.

☐ Control-C stops the capture cleanly.

# 7. Troubleshooting

| Symptom | Likely cause | Corrective action |
| --- | --- | --- |
| No /dev/cu.usbmodem* port | Dongle in DFU mode, wrong firmware, USB adapter, or cable issue | Exit bootloader mode, reconnect, verify firmware, and try another data-capable port/adapter. |
| Serial port is busy | Another program owns the device | Close Wireshark, Programmer, serial terminals, and other Python programs; then retry. |
| No module named serial | pyserial is missing from this Python environment | Run python3 -m pip install --user pyserial and verify the import. |
| No packets appear | Wrong radio channel or no nearby traffic | Confirm all nodes and the sniffer use channel 15; place the sniffer near the transmitter. |
| Only DEVICE lines appear | Firmware response does not match the expected text format | Confirm command-line sniffer firmware; inspect the raw lines for the actual format. |
| Unreadable binary characters | Binary Extcap firmware is installed | Use Nordic's Extcap utility or flash compatible command-line sniffer firmware. |
| Permission denied | Script/file access restriction | Run through python3 and confirm the serial device permissions; do not run Wireshark as sudo. |

# 8. Lab Questions

How does RSSI change as the distance between the transmitter and sniffer increases?

Does antenna orientation affect RSSI, LQI, or the number of packets observed?

What happens when the sniffer is configured for a channel different from the transmitter?

Which bytes in the captured frame correspond to the IEEE 802.15.4 frame-control and addressing fields?

How many packets were captured, and what were the minimum, maximum, and average RSSI values?

# 9. Submission

The captured_packets.txt or captured_packets.jsonl file.

A short table summarizing packet count, RSSI range, and LQI range.

Answers to the lab questions.

A brief note identifying the firmware version and selected channel.

# References

Nordic Semiconductor, nRF Sniffer for 802.15.4 User Guide. https://docs.nordicsemi.com/r/bundle/ug_sniffer_802154/

Nordic Semiconductor, IEEE 802.15.4 Sniffer sample documentation. https://github.com/nrfconnect/sdk-nrf/tree/main/samples/peripheral/802154_sniffer