TECHNICAL CODE REFERENCE

# OzturkLab nRF52840
IEEE 802.15.4 Point-to-Point Application

Detailed documentation

| Property | Value |
| --- | --- |
| Platform | Nordic nRF52840 |
| SDK | nRF Connect SDK 3.4.0 / Zephyr 4.4.0 |
| Network API | AF_PACKET, SOCK_DGRAM, IEEE 802.15.4 L2 |
| Topology | Sender 0x0001 → Receiver 0x0003 |
| Channel / PAN | 15 / 0x1234 |
| Application payload | 12-byte OzturkLab (OZ) record |
| Purpose: One source file is compiled as either a periodic sender or a packet-aware receiver. Zephyr constructs and parses the IEEE 802.15.4 MAC header; the application handles only its 12-byte payload. |  |

# 1. Executive Summary

The program implements a fixed-channel, point-to-point IEEE 802.15.4 experiment. A sender creates one application record per second, serializes it explicitly in little-endian order, and sends it to short address 0x0003. A receiver obtains payloads asynchronously through a Zephyr net_context callback, preserves per-frame RSSI and MAC-source metadata, queues the result, validates the OZ protocol, and logs the decoded fields.

## 1.1 Active execution paths

| Build role | Initialization | Steady-state path |
| --- | --- | --- |
| ROLE_SENDER | configure_ieee802154() → create_packet_socket() | run_sender() → serialize_packet() → send_oz_packet() |
| ROLE_RECEIVER | configure_ieee802154() → create_receive_context() | packet_receive_callback() → receive_queue → run_receiver() → deserialize_packet() |
| Important: receive_oz_packet() is defined but is not called by either active role. The receiver uses packet_receive_callback() because AF_PACKET recvfrom() does not expose the packet RSSI metadata needed by the experiment. |  |  |

## 1.2 System flow

SENDER THREAD
  build oz_packet → serialize 12 bytes → sendto() → IEEE 802.15.4 L2 → radio

RECEIVER NETWORK CONTEXT
  radio → IEEE 802.15.4 L2 → callback(net_pkt + metadata) → four-entry message queue

RECEIVER APPLICATION THREAD
  queue wait → payload validation → destination check → RSSI/MAC-source logging

# 2. Compile-Time Configuration

| Symbol | Current value | Effect |
| --- | --- | --- |
| NODE_ROLE | ROLE_RECEIVER | Compiles receiver-only globals and functions; change to ROLE_SENDER for the source node. |
| IEEE802154_CHANNEL | 15 | All communicating devices and the sniffer must listen on the same channel. |
| IEEE802154_PAN_ID | 0x1234 | Identifies the experiment's IEEE 802.15.4 PAN. |
| IEEE802154_TX_POWER_DBM | 0 dBm | Requested radio transmit power; support depends on the radio driver. |
| SENDER_ADDR | 0x0001 | Sender MAC short address and application source_id. |
| RECEIVER_ADDR | 0x0003 | Receiver MAC short address and application destination_id. |
| TX_PERIOD_MS | 1000 ms | Produces approximately one application transmission per second. |
| RX_QUEUE_DEPTH | 4 | At most four callback-copied frames can wait for the application thread. |
| RX_WAIT_TIMEOUT_SECONDS | 5 s | Receiver emits a status message after five seconds with no queued frame. |
| Board build rule: Compile and flash one board with NODE_ROLE set to ROLE_SENDER and the other with NODE_ROLE set to ROLE_RECEIVER. Both must share channel 15 and PAN ID 0x1234. |  |  |

# 3. Application Payload Protocol

The wire representation is exactly 12 bytes. The code deliberately does not transmit the C structure directly. serialize_packet() and deserialize_packet() make the byte layout independent of compiler padding and host alignment.

| Offset | Size | Field | Encoding / meaning |
| --- | --- | --- | --- |
| 0–1 | 2 | signature | ASCII 'O', 'Z'; rejects unrelated payloads. |
| 2 | 1 | version | Must equal PROTOCOL_VERSION (1). |
| 3 | 1 | message_type | Currently set to MESSAGE_TYPE_DATA (1); receiver does not validate it. |
| 4–5 | 2 | source_id | Unsigned 16-bit, little-endian; sender writes 0x0001. |
| 6–7 | 2 | destination_id | Unsigned 16-bit, little-endian; receiver accepts only 0x0003. |
| 8–9 | 2 | value | Signed 16-bit value stored as a little-endian 16-bit bit pattern. |
| 10–11 | 2 | sequence | Unsigned 16-bit, little-endian; starts at 1 and wraps after 65535. |

Example logical record
  signature='OZ', version=1, type=1, source=0x0001, destination=0x0003,
  value=100, sequence=1

Serialized bytes
  4F 5A 01 01 01 00 03 00 64 00 01 00

Two byte orders are intentional: Application fields use little-endian serialization. Link-layer short addresses placed in sockaddr_ll use big-endian/network order. Reversing either convention changes the decoded values or destination address.

# 4. Data Structures and Global State

## 4.1 struct oz_packet

This is the convenient in-memory representation of the application message. It contains the same logical fields as the wire record, but only the serializer defines the transmitted format.

## 4.2 struct received_frame

The callback cannot safely hand the original net_pkt to the main application loop after releasing it. It therefore copies the payload and selected metadata into received_frame:

| Field | Purpose |
| --- | --- |
| data[IEEE802154_MTU] | Bounded storage for the payload delivered by the packet-aware receive context. |
| length | Actual number of bytes copied. |
| mac_source | Decoded two-byte source address, or 0xFFFF when unavailable. |
| rssi_dbm | RSSI associated with that exact net_pkt; it uses int16_t storage. |

## 4.3 Receiver message queue

K_MSGQ_DEFINE creates a statically allocated, four-entry queue with four-byte alignment. The callback uses K_NO_WAIT, so it never blocks the network receive path. If the consumer falls behind, the newest arriving frame is dropped and a warning is logged.

## 4.4 Other globals

| Global | Role | Purpose |
| --- | --- | --- |
| iface | Both | Default Zephyr network interface used for management requests and interface index. |
| packet_socket | Sender | AF_PACKET/SOCK_DGRAM descriptor; initialized to -1. |
| receive_context | Receiver | Packet-aware Zephyr network context used for asynchronous receive. |
| tx_sequence | Sender | 16-bit sequence counter initialized to 1; post-incremented after assignment. |

# 5. Function-by-Function Reference

## 5.1 role_name() — lines 196–214

Compile-time conditional code returns the printable role name. The invalid-role branch is effectively guarded by the earlier #error and is retained defensively.

## 5.2 print_packet() — lines 218–276

Copies narrow integer fields into types suitable for variadic logging, then prints a structured record. This function is diagnostic only; it does not serialize, transmit, or validate data.

## 5.3 serialize_packet() — lines 283–322

Writes each logical field into fixed offsets. sys_put_le16() provides explicit little-endian conversion. Casting value to uint16_t preserves the two's-complement bit pattern of negative int16_t values.

## 5.4 deserialize_packet() — lines 329–405

Applies validation in this order: exact length, 'OZ' signature, field extraction, and version equality. It returns false on any failure. It does not validate message_type, source_id, destination_id, sequence continuity, or MAC/application-source agreement; destination is checked later by run_receiver().

## 5.5 configure_ieee802154() — lines 412–525

Gets the default interface and returns -ENODEV if none exists.

Brings an already-up interface down before applying configuration.

Sets channel, PAN ID, local short address, and requested transmit power through net_mgmt().

Brings the interface up and stops immediately if any request fails.

Uses LOG_* for structured messages and printk() around net_if_up() for low-level debugging.

Transmit-power behavior: IEEE802154_TX_POWER_DBM is a request to the driver. If the value is unsupported, NET_REQUEST_IEEE802154_SET_TX_POWER can fail and startup terminates. The same setting is applied to the receiver although it normally does not transmit application data.

## 5.6 create_packet_socket() — lines 532–624

Creates an AF_PACKET/SOCK_DGRAM socket for ETH_P_IEEE802154 and binds it to the configured interface index. SOCK_DGRAM is central to the design: the application supplies payload bytes and Zephyr's IEEE 802.15.4 L2 supplies the MAC header. On bind failure, the descriptor is closed and reset to -1.

## 5.7 send_oz_packet() — lines 631–750

Serializes the packet into a 12-byte local buffer.

Builds sockaddr_ll with the interface index and a two-byte destination address.

Stores the MAC destination in big-endian order with sys_put_be16().

Prints all payload bytes as TX RAW for debugging.

Calls zsock_sendto() and treats negative or short transmission as an error.

## 5.8 receive_oz_packet() — lines 757–879 (unused)

This older synchronous receive path blocks in zsock_recvfrom(), prints up to 20 bytes, validates the OZ payload, and attempts to recover the MAC source address. Because recvfrom() does not expose RSSI metadata, it stores INT8_MAX as a sentinel. The active receiver does not call this function.

Cleanup opportunity: The function contains the same received < 0 error check twice. The second check is unreachable after the first returns. Remove the duplicate if this path is retained, or remove the entire unused function after confirming it is no longer needed.

## 5.9 packet_receive_callback() — lines 887–933

This callback runs in the network receive context and must stay short. It validates status/pointer inputs, obtains per-packet RSSI, extracts the link-layer source, bounds the payload length, resets the packet cursor, copies bytes, enqueues without waiting, and always releases the net_pkt reference.

| Operation | Reason |
| --- | --- |
| net_pkt_ieee802154_rssi_dbm(pkt) | Reads RSSI belonging to the exact received frame. |
| net_pkt_lladdr_src(pkt) | Obtains the source link-layer address when present. |
| length clamp | Prevents writing past received_frame.data. |
| net_pkt_cursor_init() + net_pkt_read() | Copies payload from Zephyr's possibly fragmented packet buffers. |
| k_msgq_put(..., K_NO_WAIT) | Keeps the network callback nonblocking. |
| net_pkt_unref(pkt) | Returns the packet buffer to Zephyr on every callback path. |
| Loss behavior: When all four queue slots are occupied, the incoming frame is discarded. At high packet rates, increase RX_QUEUE_DEPTH, reduce receiver logging, or move processing to a faster consumer. |  |

## 5.10 create_receive_context() — lines 940–1017

Allocates an AF_PACKET/SOCK_DGRAM net_context, registers the callback, binds the context to the IEEE 802.15.4 interface, and registers reception again after binding. The comment states that the double registration mirrors Zephyr's packet-socket setup. Each failure releases the context and sets its pointer to NULL.

Version sensitivity: The two net_context_recv() registrations are unusual at application level. Preserve them for the tested NCS 3.4.0 behavior, but re-check the API requirements when moving to another Zephyr/NCS version.

## 5.11 run_sender() — lines 1024–1130

Initializes value to 100 and then repeats forever: clear the structure, populate the OZ fields, assign and increment the sequence, print the logical packet, send it to MAC destination 0x0003, increment value, and sleep one second. Transmission failures are logged but do not terminate the loop.

## 5.12 run_receiver() — lines 1137–1228

Waits up to five seconds for a queued received_frame. Timeout is informational. Other queue errors are logged. Each frame is hex-dumped, passed to deserialize_packet(), filtered by application destination, and logged with MAC source and RSSI. The loop does not verify that application source_id equals MAC source.

## 5.13 main() — lines 1235–1345

Prints the effective configuration, initializes the radio/MAC, creates the role-specific endpoint, and enters the corresponding infinite loop. Preprocessor selection means the unused role's operational code is not part of the resulting image.

Exit-status issue: Initialization failures return 0 from main(), conventionally indicating success. Returning the negative error or EXIT_FAILURE would make automated tests and supervisors detect startup failure correctly.

# 6. Detailed Runtime Sequences

## 6.1 Sender startup and transmit cycle

| Order | Action | Failure behavior |
| --- | --- | --- |
| 1 | Print effective role/configuration | Continues |
| 2 | Configure interface and bring it up | Logs and exits main |
| 3 | Create and bind packet socket | Logs and exits main |
| 4 | Construct sequence N and value V | No dynamic allocation |
| 5 | Serialize 12-byte OZ payload | Deterministic offsets |
| 6 | sendto() to MAC 0x0003 | Logs error; next cycle continues |
| 7 | Increment value and sleep 1000 ms | Loop repeats indefinitely |

## 6.2 Receiver startup and receive cycle

| Order | Context | Action |
| --- | --- | --- |
| 1 | Main thread | Configure interface and create bound receive context. |
| 2 | Network context | Callback receives net_pkt, captures RSSI/source/payload, and enqueues a copy. |
| 3 | Main thread | Wait up to five seconds for a received_frame. |
| 4 | Main thread | Reject wrong size, signature, or version. |
| 5 | Main thread | Reject application destination other than 0x0003. |
| 6 | Main thread | Log MAC source, RSSI, and decoded application fields. |
| 7 | Both | Continue indefinitely; callback may drop frames if queue is full. |

# 7. Addressing and Filtering

| Layer | Sender value | Receiver handling |
| --- | --- | --- |
| PHY channel | 15 | Interface accepts traffic only on configured channel. |
| PAN ID | 0x1234 | Configured in IEEE 802.15.4 interface; L2 behavior depends on driver/filtering configuration. |
| MAC short address | Source 0x0001; destination 0x0003 | Local short address is 0x0003; callback reports MAC source when available. |
| Application source_id | 0x0001 | Decoded and printed, but not validated. |
| Application destination_id | 0x0003 | Explicitly checked; mismatches are ignored. |
| Defense-in-depth recommendation: For experiments that require identity integrity, validate message_type, application source_id, expected MAC source, and equality between the MAC and application source identities. |  |  |

# 8. Logging and Observable Behavior

| Message | Meaning |
| --- | --- |
| Network interface found | A default Zephyr net_if exists. |
| SET_* failed | The driver rejected or failed a management operation. |
| IEEE 802.15.4 interface UP | Radio/MAC interface initialization completed. |
| TX RAW | The sender's 12 serialized payload bytes before L2 framing. |
| RX | Up to the first 20 received payload bytes. |
| Non-OZ packet ignored | Length/signature/version validation failed. |
| RX queue full; packet dropped | Callback could not enqueue because four entries were already waiting. |
| RSSI = unavailable | Zephyr returned IEEE802154_MAC_RSSI_DBM_UNDEFINED. |
| No packet callback in the last 5 seconds | No queued frame arrived during the receiver wait interval. |

# 9. Error Handling and Return Values

| Area | Current behavior | Implication |
| --- | --- | --- |
| Management/setup | Returns the first Zephyr error code | Initialization stops at the failed operation. |
| Socket send | Returns -errno or -EIO | Sender logs it and continues later cycles. |
| Receiver callback | Drops invalid status/null packets after unref | No application-thread notification. |
| Queue overflow | Drops newest frame and logs warning | Measured packet reception can be biased under load. |
| Payload validation | Returns false / skips frame | Foreign and malformed payloads do not reach application logging. |
| Main startup failure | Returns 0 | External tools may incorrectly record a successful exit. |

# 10. Engineering Review and Recommended Improvements

| Priority | Observation | Recommended change |
| --- | --- | --- |
| High | main() returns 0 on initialization errors | Return a nonzero failure status or the original error. |
| High | Queue depth is only four and logging is relatively expensive | Measure queue overflow; tune depth and logging for the expected packet rate. |
| Medium | receive_oz_packet() is unused and duplicates an error check | Remove it or mark/document it as an alternate diagnostic path. |
| Medium | message_type and source identities are not validated | Validate type, source_id, MAC source, and their consistency. |
| Medium | Sequence gaps and duplicates are not detected | Track the previous sequence per source and report loss, duplicate, and out-of-order events. |
| Medium | No LQI is retained | If available in net_pkt metadata for this stack, add LQI to received_frame and logs. |
| Low | Mixed printk and LOG output | Use one logging mechanism after bring-up debugging is complete. |
| Low | TX value and sequence silently wrap | Document wrap semantics or use wider counters if the experiment runs for long periods. |

# 11. Suggested Receiver Validation Logic

The following conceptual checks would make the experiment more rigorous without changing the wire format:

if (packet.message_type != MESSAGE_TYPE_DATA) ignore;
if (packet.source_id != SENDER_ADDR) ignore;
if (frame.mac_source != SENDER_ADDR) warn/ignore;
if (packet.source_id != frame.mac_source) report identity mismatch;
compare packet.sequence with previous_sequence to estimate loss and duplicates;

# 12. Build and Deployment Checklist

Sender build: NODE_ROLE = ROLE_SENDER; local address resolves to 0x0001.

Receiver build: NODE_ROLE = ROLE_RECEIVER; local address resolves to 0x0003.

Both builds: channel 15, PAN ID 0x1234, compatible Zephyr/NCS configuration.

Confirm the selected transmit power is supported by the nRF52840 driver.

Verify required Zephyr networking, IEEE 802.15.4, packet socket, and logging options in prj.conf.

Flash the two role-specific images to different boards.

Verify startup logs show interface UP and the correct role/address.

Place a sniffer on channel 15 and compare observed MAC frames with TX/RX logs.

Configuration-file boundary: Only main(4).c was supplied. Exact prj.conf, board overlay, Kconfig, CMake, and driver requirements cannot be confirmed from this source file alone.

# 13. Quick Reference

| Task | Change / observe |
| --- | --- |
| Change role | Edit only NODE_ROLE and rebuild. |
| Change channel | Set IEEE802154_CHANNEL to 11–26 on every participant and sniffer. |
| Change PAN | Set IEEE802154_PAN_ID consistently. |
| Change TX power | Set IEEE802154_TX_POWER_DBM to a driver-supported value. |
| Change sending rate | Adjust TX_PERIOD_MS. |
| Handle bursts | Increase RX_QUEUE_DEPTH and reduce logging overhead. |
| Identify packet loss | Add per-source sequence tracking. |
| Observe RSSI | Use the receiver callback path; the blocking recvfrom path cannot provide it. |

# 14. Source Scope

This document describes the attached 1,345-line source file main(4).c. Line references correspond to that supplied version. The explanation is based on the visible program logic and comments; behavior that depends on prj.conf, devicetree, radio-driver implementation, or later Zephyr changes should be verified in the complete build project.